# SHIELDGUARD INC. - SECURE NETWORK TOPOLOGY ( SUMMARY)

Objective: Design a secure, scalable network for a small office that enables segmented access for employees, guests, and internet-facing services.

## Network Zones & Addressing:

| Zone | VLAN | Subnet | Interface |
| --- | --- | --- | --- |
| Inside LAN | 10 | 192.168.10.0/24 | GigabitEthernet0/1 |
| Guest Wi-Fi | 20 | 192.168.20.0/24 | GigabitEthernet0/2 |
| DMZ | 30 | 192.168.30.0/24 | GigabitEthernet0/3 |
| Outside | — | 203.0.113.0/24 | GigabitEthernet0/0 |

## Security Features:

- Firewall (Cisco ASA): Defines security levels: inside (100), guest (50), dmz (25), outside (0)

- Access Control Lists (ACLs):

- Inside LAN → Permit all

- Guest → Deny access to Inside LAN

- Outside → Permit HTTP/HTTPS to DMZ

- NAT Rules:

- Inside & Guest to Internet: PAT

- Static NAT for DMZ Web Server (203.0.113.2)

## Wireless Configuration:

- SSIDs: Office_WiFi (VLAN 10), Guest_WiFi (VLAN 20)

- WPA2 Encryption, strong passwords

- VLAN tagging enabled

## Testing Summary:

- Office to Internet ✅

- Guest to Internet ✅

- Guest to Office ❌ (Blocked)

- Outside to Web Server ✅

- VLAN Isolation ✅

Conclusion: This setup ensures strong segmentation, secure internet access, public-facing services via DMZ, and future scalability. Ideal for modern small office needs.

Prepared by: [Your Name]

Date: [Insert Date]

Network Topology Diagram

Below is a visual representation of the secure network design:

Cisco ASA CLI Firewall Configuration Examples

! OUTSIDE Interface
interface GigabitEthernet0/0
 nameif outside
 security-level 0
 ip address 203.0.113.1 255.255.255.0
 no shutdown

! INSIDE Interface
interface GigabitEthernet0/1
 nameif inside
 security-level 100
 ip address 192.168.10.1 255.255.255.0
 no shutdown

! GUEST Interface
interface GigabitEthernet0/2
 nameif guest
 security-level 50
 ip address 192.168.20.1 255.255.255.0
 no shutdown

! DMZ Interface
interface GigabitEthernet0/3
 nameif dmz
 security-level 25
 ip address 192.168.30.1 255.255.255.0
 no shutdown

! Access Rules
access-list inside_access_in extended permit ip any any
access-group inside_access_in in interface inside

access-list guest_access_in extended deny ip any 192.168.10.0 255.255.255.0
access-list guest_access_in extended permit ip any any
access-group guest_access_in in interface guest

access-list outside_access_in extended permit tcp any host 192.168.30.2 eq 80
access-list outside_access_in extended permit tcp any host 192.168.30.2 eq 443
access-group outside_access_in in interface outside

## Testing & Validation

Test Cases Conducted:
- Ping from Inside LAN to Internet: Success
- Ping from Guest Wi-Fi to Internet: Success
- Ping from Guest to Inside: Blocked (Expected)
- Web access from Outside to DMZ (HTTP/HTTPS): Success
- Confirmed VLAN isolation using traceroute and ping between zones
- Verified NAT translation and ACLs using ASA 'show nat', 'show access-list'

Conclusion:
All configured zones function independently with strict firewall segmentation. The implementation was validated against expected behavior for each test case.

## Scalability Considerations

This design can scale by:
- Adding new VLANs on the managed switch for additional departments
- Defining new NAT/PAT rules for expanded internet access
- Adding more DMZ services using static NAT mappings
- Leveraging ASA object groups for simplified ACL management